Synamedia MEG setup – HSN

HSN1 HD MP4 ASI Out

Task- Configuring Synamedia MEG IRDs

Condition- Service Routing HSN1 HD MP4 - ASI OUT

Standard- How to configure a Synamedia MEG IRD for Service Routing

Action Items:

- Service Routing HD MP4

  - Begin by Browsing to IP of MEG IRD (Default ip - 192.168.2.20)

    - Enter UN & PW (see figure 1) (UN: Admin ; PW: Password)

figure 1 - login

- Click on Add New Channel (see figure 2)

figure 2 – add channel

  - Choose which option you would like to do

    - Select Service Routing for ASI setup (see figure 3)

figure 3 – Service Routing

  - Service Routing – ASI

    - Select Use Existing Input

    - Input Selection - DVB-S2:DVBS2 1 (4.08GHz)

    - Select which Service ID you would like to Route by choosing the service in the Dropdown Menu - (23) for HSN2 HD (see figure 4)

figure 4 – configure Service Routing options

- Descrambling - Disabled

- Click on Output next

  - Select Type

    - Select ASI

      - Select which ASI port you would like to use (Asi1)

      - Leave remaining fields as default values

  - Tick Service ID box

  - Enter Service ID – 20 (see figure 5)

Figure 5 – configure output

- Click on Create in upper right corner (see figure 6)

Figure 6 – create and finish

- You should see a popup box that says “Channel activated successfully”

- Setup complete – There should now be 1 channel with a green check mark (see figure 7)

Figure 7 – Channel Created

*Advanced View configurations are beyond the scope of this document.